Supplementary Information for:

Conformal Slicing Strategy for Robot-assisted Direct Ink Writing on Unstructured Curved Surfaces

Peng Chen1, Qinxiang Liu1, Jun Ren1, 2, Yanqiu Chen1, 2, Jian Zhang3,*, Yu Liu1, 2, *

1. School of Mechanical Engineering, Jiangnan University, 214122 Wuxi, China.

2. Jiangsu Key Lab of Advanced Food Manufacturing Equipment and Technology, Jiangnan University, 214122 Wuxi, China.

3. Shanghai Institute of Ceramics, Chinese Academy of Sciences, 201899, Shanghai, China.

*Corresponding author. E-mail: yuliu@jiangnan.edu.cn; Jianzhang@mail.sic.ac.cn

S1. Pseudocode of self-intersection processing

Algorithm 1: Self-intersection processing

Input: The self-intersection triangular mesh surface S0 contains array F0, and array V0. offset distance doffset. Output: The Non-self-intersection triangular mesh surface S1, contains array F1, and array V1.

- array SF0, array SV0 ← selfintersect(F0, V0) - if (length(SV0)＞length(V0))then - array D ← Calculate the distance from points SV0 to the original surface S0. - for (i = 1:length(D) do - if (Di＞doffset) - array Flagi ← 0; - else - array Flagi ← 1; - end if - end for - for (j = 1:length(SF0) do - if (Flag(SF0(j,1)) && Flag(SF0(j,2))&&Flag(SF0(j,3))) then - Delete triangle SF0(j,:); - end if - end for - Delete the points in SV0 that do not exist in SF0's index, Update SF0; - F1←SF0; V1←SV0 - return S1 - else if (length(SV0) == length(V0)) then - F1← F0, V1← V0, - strcut S1 ←F1, V0

- return S1

- else

- return 0

- end if

S2. Pseudocode of the model and surface intersection calculation

Algorithm 2: Model and surface intersection calculation

Input: The slicer triangular mesh surface Ss contains array Fs, and array Vs. The STL format model Sm contains array Fm, and array Vm. Output: The trimmed triangular mesh surface St, contains array Ft, and array Vt.

- cellarray Esm , array Vsm ← SurfaceIntersection(Ss, Sm). - array Fsi , array Vsi ← TriangleDivision(Esm, Vsm, Ss). - strcut Ssi ←Fsi, Vsi. - array Csi TriangleCentroidCalculation(Ssi). - for (j = 1 : length(Esm))do - array InTriFlag ← InPolygonDetection(Esm{j},Vsm,Csi). - array MatrixInTri(:,j) ←InTriFlag. - end for - array NedDelFlag ← 1 - mod(sum(MatrixInTri, 2),2). - Fs ← Fs (NedDelFlag == 0, :). - update Vs. - Ft← Fs, Vt← Vs. - strcut St ←Ft, Vt. - return St

S3. Pseudocode of continuous path drawn in one stroke

Algorithm 3: Continuous path drawn in one stroke

Input: The isoline contours on surface C contains array Ec, and array Vc. Output: The continuous path P, contains array Vp.

- graph GC ← createContourGraph(C). - graph TC ← minspantree(GC). - graph TR ← graph TC. - while(true) - cellarray RegionsR3 ← GetDegreeMore2Nodes (TR) - if(length(RegionsR3)=0)then - graph RegionR ← FermatsSpiralConnection(TR). - Vp ← Graph2Path(RegionR). - P ←Vp - return P. - else - cellmatrix RegionR2 ← GetDegreeLess2NodesConnected (TR) - for (i = 1 : length(RegionR2(1,:))) do - for (j = 1 : length(RegionR2(:,1))) do - graph Crj ←RegionR2{i,j}. - graph Regionj ← FermatsSpiralConnection(Crj). - graph Ri,j← Regioni,j. - end for - end for - graph GR ← createRegionGraph(R). - graph TR ← minspantree(GR). - cellArray RegionR1 ← GetDegree1NodesConnected (TR) - for(index = 1 : length(RegionsR3)) do - RegionsR3'{index} ← RegionMerge(RegionR1, RegionsR3) - end for - R ← UpdateRegionGraph( RegionsR3', R). - graph GR ← createRegionGraph(R). - graph TR ← minspantree(GR). - end if - end while

S4. Convert print path points into executable files for the robotic arm

Fig.S1. Path points and marker points under the model coordinate system and the robot arm coordinate system are used for coordinate transformation and pose generation. (a) Substrate model and marker points in the CAD Coordinate System and Ppath(i) with reference vector Xf ; (b) Physical substrate and marker points in the Robotic Arm Coordinate System and TCP pose at Ppath(i);

As shown in Fig.S1(a-b), in this study, three non-collinear marker points PCAD, were designed on the substrate model, while three corresponding marker points Probot were fabricated on the physical substrate and located by move robot’s TCP to those points. Based on the coordinate values of these points, the translation and rotation matrices were calculated to obtain the spatial translation and rotation transformation function ftrans, The calculation of the transformation function is based on Eq. S1-4. Then, the path points were subjected to an overall transformation using this transformation function, thereby converting the coordinate values of the printing path points into the robotic arm coordinate system. The TCP pose is calculated from the normal information of the path points and a given reference axis, as specified in Eq. S5.

Xrobot= Probot2-Probot1Zrobot=corssXrobot,Probot3-Probot1Yrobot=corssZrobot,XrobotRrobot=Xrobot,Yrobot,Zrobot#S1

In Eq.S1 where Xrobot denotes the vector formed by two points in the robotic coordinate system, which serves as the X-axis, Probot denotes the set of marked points in the robotic coordinate system, Zrobot denotes the vector in the robotic coordinate system, which serves as the Z-axis, Yrobot denotes, the vector in the robotic coordinate system, which serves as the Y-axis, corss denotes cross product of vectors and Rrobot denotes spatial transformation matrix established based on three vectors in robotic coordinate system.

XCAD= PCAD2-PCAD1ZCAD=corssXCAD,PCAD3-PCAD1YCAD=corssZCAD,XCADRCAD=XCAD,YCAD,ZCAD#S2

In Eq.S2, where XCAD denotes the vector formed by two points in the CAD coordinate system, which serves as the X-axis, PCAD denotes the set of marked points in the CAD system, ZCAD denotes the vector in the CAD coordinate system, which serves as the Z-axis, YCAD denotes, the vector in the CAD coordinate system, which serves as the Y-axis, corss denotes cross product of vectors and RCAD denotes spatial transformation matrix established based on three vectors in CAD coordinate system.

Rtrans=RrobotRCADTtrans= Probot1-Rtrans*PCAD1#S3

In Eq. S3, where Rtrans denotes spatial rotation matrix for transformation from the CAD coordinate system to the robotic arm coordinate system, Ttrans denotes Spatial translation matrix for transformation from the CAD coordinate system to the robotic arm coordinate system.

Mtrans=Rtrans,Ttransftrans:Mtrans*Ppath#S4

In Eq.S4, where Mtrans denotes spatial rotation and translation matrix for transformation from the CAD coordinate system to the robotic coordinate system, ftrans denotes function for transforming path points from the CAD coordinate system to the robotic coordinate system, Ppath denotes path points in CAD coordinate system.

Zi=-ni-niYi=Zi×XfZi×XfXi= Yi×ZiYi×ZiRi=Xi,Yi,Ziρi=rot2axangRiposei=xi,yi,zi,ρix,ρiy,ρiz#S5

In Eq.S1 where Zi denotes the Z-axis of the TCP at path i calculated from the normal vector of path point i, ni denotes the normal vector of path point i, Yi denotes the Y-axis of the TCP at path i, Xi denotes the X-axis of the TCP at path i, Ri denotes the rotation matrix of the TCP at path point i, ρi denotes axis-angle rotation of the TCP at path point i, posei denotes the pose of the TCP at path point i.

S5. Ink preparation process

The preparation process of Ink 1 was as follows: the SE1700 PDMS matrix and curing agent were mixed in a material bottle at a mass ratio of 10:1, and homogenously stirred at a rotational speed of 1500 r/min using a planetary mixer (Model QM3SP2, Shenzhen Zhongyan Yichuang Technology Co., Ltd.); subsequently, the mixture was subjected to centrifugal defoaming at 3500 r/min for 5 min with a centrifuge (Model TG1650-WS, Shanghai Luxiangyi Centrifuge Instrument Co., Ltd.) to remove internal air bubbles, thus completing the preparation of Ink 1.

For the preparation of Ink 2, Sylgard 184 PDMS and curing agent were mixed in a material bottle at a mass ratio of 10:1 with the addition of carbon black at a mass fraction of 8%. The mixture was then homogenously stirred at 1500 r/min using a planetary mixer (Model QM3SP2, Shenzhen Zhongyan Yichuang Technology Co., Ltd.); thereafter, centrifugal defoaming was performed at 3500 r/min for 5 min with a centrifuge (Model TG1650-WS, Shanghai Luxiangyi Centrifuge Instrument Co., Ltd.) to eliminate internal air bubbles, and the preparation of Ink 2 was thus finished.

Movie S1. Needle printing pose adjustment (MP4, 23.0 MB).

Here is a demonstration of the movement effect of the printing nozzle on curved surfaces after its pose adjustment.

Movie S2. Continuous filling paths on different unstructured surfaces (MP4, 38.9 MB).

This is a path animation demonstration of one-stroke complete filling on various unstructured curved surfaces.

Movie S3. Continuous filling paths on different unstructured surfaces (Lattice Structure Embeded) (MP4, 29.0 MB).

This is a path animation demonstration of the partial embedding and filling of lattice structures with a one-stroke approach on various unstructured curved surfaces.

Movie S4. Muti-layer continous path printing (MP4, 43.1MB).

This is a video demonstrating the one-stroke complete filling of complex structures on multi-layered complex curved surfaces, which includes a full path animation demonstration and synchronous multi-perspective physical printing footage. The video has been speeded up due to the long printing duration.

Movie S5. Conductivity test of multilayer functional structures (MP4, 20.6 MB).

This is a video demonstrating the electrical conductivity of the printed multi-layered functional materials on curved surfaces by means of a digital multimeter.